Patching Runbook

Network Switches

MSP Managed Infrastructure

| Document ID | MSP-RB-SW |
| --- | --- |
| Device Type | Network Switches |
| Version | 1.0 |
| SOP Reference | MSP-PAT-002 |

CR #: _________________    Runbook ID: RB-_________________

SECTION 1 – RUN INFORMATION

| Customer Name | Acme Corporation |
| --- | --- |

| Device Type | Network Switches |
| --- | --- |

| Device Hostname(s) / IP(s) |  |
| --- | --- |

| Current Version (Pre-Patch) |  |
| --- | --- |

| Target Version (Post-Patch) |  |
| --- | --- |

| Patch Source / URL |  |
| --- | --- |

| Checksum (MD5 / SHA256) |  |
| --- | --- |

| Maintenance Window |  |
| --- | --- |

| Executing Engineer |  |
| --- | --- |

| Customer POC (Name + Phone) | Jane Smith — +1 (555) 123-4567 |
| --- | --- |

| Bridge / Teams Call Link |  |
| --- | --- |

| CR Number |  |
| --- | --- |

SECTION 2 – PRE-PATCH STATE CAPTURE

IMPORTANT: Run all commands below BEFORE making any changes. Paste outputs or screenshot references in the Captured Value column.

| Item | Command / Source | Captured Value |
| --- | --- | --- |
| Current IOS / firmware version | show version |  |
| Spanning tree root / topology | show spanning-tree ¦ show spanning-tree root |  |
| CDP / LLDP neighbors | show cdp neighbors detail / show lldp neighbors |  |
| Interface status (all ports) | show interfaces status / show ip interface brief |  |
| VLAN configuration | show vlan brief |  |
| Routing table (if L3 switch) | show ip route |  |
| EtherChannel / port-channel status | show etherchannel summary |  |

SECTION 3 – PREREQUISITES CHECKLIST

- Approved Change Request and customer written approval on file

- Running-config backup saved: copy running-config tftp://<server>/<hostname>-pre-patch.cfg

- IOS / firmware image verified on TFTP/SCP server, MD5 checksum confirmed

- Network topology diagram reviewed — uplinks, redundant paths, and spanning tree root identified

- Out-of-band console access confirmed (serial console or OOB management network)

- For stacked switches: stack upgrade procedure reviewed (all members upgrade simultaneously)

SECTION 4A – PRE-PATCH STEPS

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 1 | Notify NOC — maintenance window open | NOC acknowledged |  |
| 2 | Suppress monitoring alerts for target switches | Alerts muted |  |
| 3 | Save running-config: copy running-config tftp://<server>/<hostname>-pre-patch.cfg | Config backed up |  |
| 4 | Confirm console / OOB access | Console access verified |  |
| 5 | Stage firmware image to switch flash: copy tftp://<server>/<image>.bin flash: | Image in flash |  |
| 6 | Verify image MD5 on switch: verify /md5 flash:<image>.bin | MD5 matches |  |

SECTION 4B – PATCH EXECUTION STEPS

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 7 | Set new boot image: boot system flash:<new-image>.bin | Boot var set |  |
| 8 | Verify boot statement: show boot / show bootvar | Correct image shown |  |
| 9 | For Cisco IOS-XE software install mode: install add file flash:<image>.bin install activate (This will reload the switch) | Install activated |  |
| 10 | For traditional IOS bundle mode: reload at <HH:MM> (scheduled within maintenance window) OR reload (immediate) | Reload initiated |  |
| 11 | Monitor console — watch boot sequence for errors | No boot errors |  |
| 12 | Switch returns to operational state | Switch online |  |
| 13 | Verify new IOS version: show version ¦ include Version | New version confirmed |  |
| 14 | For install mode — commit the install: install commit | Install committed |  |

SECTION 4C – POST-PATCH STEPS

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 15 | Verify spanning tree topology unchanged: show spanning-tree — compare root and port states to baseline | STP topology: stable |  |
| 16 | Verify all uplinks and access ports: show interfaces status | All expected ports: connected |  |
| 17 | Verify VLANs intact: show vlan brief | VLANs: correct |  |
| 18 | Verify routing (if L3): show ip route — compare to baseline | Routing: stable |  |
| 19 | Run ping tests from key network segments through the switch | Connectivity: pass |  |
| 20 | De-suppress monitoring alerts | Alerts active |  |
| 21 | Notify NOC — maintenance window closed | NOC notified |  |
| 22 | Customer POC sign-off | Customer confirmed OK |  |

SECTION 5 – VALIDATION TEST RESULTS

| Validation Test | Method / Command | Pass / Fail | Initials |
| --- | --- | --- | --- |
| New IOS / firmware version confirmed | show version ¦ include Version |  |  |
| Spanning tree root unchanged | show spanning-tree root |  |  |
| All uplinks connected | show interfaces status ¦ include connected |  |  |
| EtherChannel bundles up | show etherchannel summary |  |  |
| VLANs intact | show vlan brief |  |  |
| CDP / LLDP neighbors match baseline | show cdp neighbors / show lldp neighbors |  |  |
| Routing table stable (L3 switch) | show ip route — compare to pre-patch |  |  |
| End-to-end connectivity test | Ping across VLANs from test hosts |  |  |

SECTION 6 – ROLLBACK LOG (complete only if rollback invoked)

| Rollback Invoked? | [ ] Yes [ ] No |
| --- | --- |

| Rollback Trigger / Reason |  |
| --- | --- |

| Rollback Start Time |  |
| --- | --- |

| Rollback End Time |  |
| --- | --- |

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 1 | If switch fails to boot: access ROMMON via console (Ctrl+Break at boot) | ROMMON access |  |
| 2 | Boot prior image from ROMMON: ROMON> boot flash:<prior-image>.bin | Prior IOS boots |  |
| 3 | Restore config from backup: copy tftp://<server>/<hostname>-pre-patch.cfg running-config copy running-config startup-config | Config restored |  |
| 4 | For install mode rollback: install rollback to committed | Rolled back to committed |  |
| 5 | Verify switch operation and notify Patching Manager | Switch operational; PM notified |  |

| Rollback Outcome | [ ] Successful [ ] Partially Successful [ ] Failed — Escalated |
| --- | --- |

SECTION 7 – AS-BUILT DOCUMENTATION

| Field | Value |
| --- | --- |
| IOS Version (Before) |  |
| IOS Version (After) |  |
| Install Mode (Bundle / Install) |  |
| Actual Reload Time |  |
| Actual Start Time |  |
| Actual End Time |  |
| Switches Patched (list) |  |

## Post-Patch Command Output

Paste output of: show version  |  show spanning-tree  |  show interfaces status  |  show ip route

SECTION 8 – SIGN-OFF

| Role | Name | Date / Time | Initials |
| --- | --- | --- | --- |
| Executing Engineer |  |  |  |
| Patching Manager |  |  |  |
| Customer POC | Jane Smith |  |  |

SECTION 9 – ISSUES AND LESSONS LEARNED