Quick Reference Card — Cisco IOS / IOS-XE

Essential Patching Commands — Field Engineer Reference

Technician

| Document ID | MSP-TEC-002 |
| --- | --- |
| Category | Technician |
| Version | 1.0 |
| Audience | Field / Infrastructure Engineer |

# Command Reference  —  Acme Corporation

Commands most commonly needed during patching maintenance windows. Keep this open alongside the runbook during the maintenance window.

| Action | Command |
| --- | --- |
| Show IOS version | show version ¦ include Version |
| Show boot variable | show boot |
| Verify image MD5 | verify /md5 flash:<image>.bin |
| Copy image from TFTP | copy tftp://<server>/<image>.bin flash: |
| Set boot variable | boot system flash:<image>.bin (in config t) |
| Save config | copy running-config startup-config |
| IOS-XE: add image | install add file flash:<image>.bin |
| IOS-XE: activate | install activate |
| IOS-XE: commit | install commit |
| IOS-XE: rollback | install rollback to committed |
| Show spanning tree | show spanning-tree |
| Show interfaces | show interfaces status |
| Show VLANs | show vlan brief |
| Show CDP neighbors | show cdp neighbors detail |
| ROMMON boot | ROMMON> boot flash:<image>.bin |

# Pre-Patch Checks — Quick List

- Approved CR on file — confirmed

- Customer POC notified and available

- Backups/snapshots current (< 24 hours)

- OOB / console access confirmed

- Patch bundle downloaded and checksum verified

- Monitoring alerts suppressed

- NOC notified — window open

# Post-Patch Checks — Quick List

- New version/build confirmed

- All services / interfaces up

- HA / cluster sync confirmed (if applicable)

- Validation tests passed

- Monitoring alerts re-enabled

- NOC notified — window closed

- Customer POC sign-off obtained

- Runbook completed and filed

# Escalation Quick Reference

| Situation | Who to Call | When |
| --- | --- | --- |
| Rollback not working within 30 min | Patching Manager | Immediately |
| Device fails to boot after patch | Patching Manager + Vendor TAC | Within 15 min of confirmed boot failure |
| Data appears inaccessible (SAN/SQL) | Patching Manager — P1 Incident | Immediately |
| Customer escalation during window | Account Manager | Immediately |
| Maintenance window overrunning by > 50% | Patching Manager | Before exceeding window end time |