Patching Runbook

SAN / Storage Array

MSP Managed Infrastructure

| Document ID | MSP-RB-SAN |
| --- | --- |
| Device Type | SAN / Storage Array |
| Version | 1.0 |
| SOP Reference | MSP-PAT-002 |

CR #: _________________    Runbook ID: RB-_________________

SECTION 1 – RUN INFORMATION

| Customer Name | Test Customer Inc. |
| --- | --- |

| Device Type | SAN / Storage Array |
| --- | --- |

| Device Hostname(s) / IP(s) |  |
| --- | --- |

| Current Version (Pre-Patch) |  |
| --- | --- |

| Target Version (Post-Patch) |  |
| --- | --- |

| Patch Source / URL |  |
| --- | --- |

| Checksum (MD5 / SHA256) |  |
| --- | --- |

| Maintenance Window |  |
| --- | --- |

| Executing Engineer |  |
| --- | --- |

| Customer POC (Name + Phone) | John Doe — +1 (555) 000-1234 |
| --- | --- |

| Bridge / Teams Call Link |  |
| --- | --- |

| CR Number |  |
| --- | --- |

SECTION 2 – PRE-PATCH STATE CAPTURE

IMPORTANT: Run all commands below BEFORE making any changes. Paste outputs or screenshot references in the Captured Value column.

| Item | Command / Source | Captured Value |
| --- | --- | --- |
| SAN firmware version (both controllers) | SAN GUI → System → Firmware OR CLI: show firmware |  |
| Controller A status | SAN GUI → Controllers |  |
| Controller B status | SAN GUI → Controllers |  |
| Volume / LUN presentation status | SAN GUI → Volumes OR esxcli storage vmfs list |  |
| Host multipath status (from ESXi) | esxcli storage nmp device list |  |
| Active I/O / performance baseline | SAN GUI → Performance Monitor |  |
| Last successful backup | Backup console — verify all SAN volumes backed up |  |

SECTION 3 – PREREQUISITES CHECKLIST

- Vendor release notes reviewed — compatibility with connected ESXi and OS versions confirmed

- Firmware bundle downloaded from vendor support portal, checksum verified

- Both SAN controllers healthy (no degraded/failed disks, no rebuilds in progress)

- Customer application team on standby — app quiesce may be required

- All backup jobs targeting SAN volumes paused or completed

- Multipath software (MPIO) confirmed active on all connected hosts

SECTION 4A – PRE-PATCH STEPS

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 1 | Notify NOC — maintenance window open | NOC acknowledged |  |
| 2 | Suppress monitoring alerts for SAN and connected hosts | Alerts muted |  |
| 3 | Pause all backup jobs targeting SAN volumes | Backup jobs paused |  |
| 4 | Coordinate with customer app team — quiesce applications if required by vendor NDU procedure | App I/O quiesced (or confirmed NDU-safe) |  |
| 5 | Log in to SAN management GUI and CLI | Management access confirmed |  |
| 6 | Upload firmware bundle to SAN management plane via GUI upload tool | Bundle uploaded |  |
| 7 | Verify bundle integrity via SAN GUI checksum utility | Checksum: pass |  |

SECTION 4B – PATCH EXECUTION STEPS

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 8 | Initiate NDU (Non-Disruptive Upgrade) via SAN GUI: System → Firmware → Apply / Install | NDU begins |  |
| 9 | Monitor Controller B firmware update (standby controller upgrades first) | Controller B: upgraded and online |  |
| 10 | Automatic controller failover occurs — Controller B becomes active (Expected — I/O continues via multipath) | Failover: complete, I/O continuing |  |
| 11 | Monitor Controller A firmware update | Controller A: upgraded and online |  |
| 12 | Automatic failback — Controller A returns to preferred role (Behavior varies by vendor — some remain on Controller B) | Both controllers: new firmware |  |
| 13 | Confirm both controllers show new firmware version in GUI | Firmware versions match target |  |
| 14 | Resume I/O fully; validate volume presentation from all connected hosts | All LUNs presented correctly |  |

SECTION 4C – POST-PATCH STEPS

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 15 | Re-enable backup jobs; run manual test backup of key volume | Test backup: successful |  |
| 16 | Check host multipath status — confirm all paths active: esxcli storage nmp device list ¦ grep -A5 <device> | All paths: active |  |
| 17 | Review SAN performance monitor — confirm I/O latency normal | Latency: within baseline |  |
| 18 | De-suppress monitoring alerts | Alerts active |  |
| 19 | Notify NOC — maintenance window closed | NOC notified |  |
| 20 | Customer POC sign-off | Customer confirmed OK |  |

SECTION 5 – VALIDATION TEST RESULTS

| Validation Test | Method / Command | Pass / Fail | Initials |
| --- | --- | --- | --- |
| Controller A firmware version | SAN GUI → System → Firmware |  |  |
| Controller B firmware version | SAN GUI → System → Firmware |  |  |
| Both controllers: online status | SAN GUI → Controllers |  |  |
| All volumes / LUNs presented to hosts | SAN GUI → Volumes ¦ esxcli storage vmfs list |  |  |
| Multipath: all paths active on all hosts | esxcli storage nmp device list |  |  |
| I/O latency within normal baseline | SAN GUI → Performance |  |  |
| Test backup successful | Backup console |  |  |
| No disk shelf / drive errors | SAN GUI → Events / Alerts |  |  |

SECTION 6 – ROLLBACK LOG (complete only if rollback invoked)

| Rollback Invoked? | [ ] Yes [ ] No |
| --- | --- |

| Rollback Trigger / Reason |  |
| --- | --- |

| Rollback Start Time |  |
| --- | --- |

| Rollback End Time |  |
| --- | --- |

| # | Step | Expected Result | Initials / Time |
| --- | --- | --- | --- |
| 1 | If NDU fails mid-upgrade: do NOT power-cycle. Contact vendor TAC immediately with case open | TAC engaged |  |
| 2 | If a controller fails to rejoin after upgrade: follow vendor controller replacement / recovery KB | Controller recovery initiated |  |
| 3 | If volume presentation is lost: check zoning and host bus adapter (HBA) status; rescan storage from host | Volumes re-presented |  |
| 4 | If rollback firmware is supported by vendor: install via GUI → Firmware → Previous Version | Rollback firmware installed |  |
| 5 | Notify Patching Manager and customer POC; open P1 incident if data is inaccessible | Stakeholders notified; incident open |  |

| Rollback Outcome | [ ] Successful [ ] Partially Successful [ ] Failed — Escalated |
| --- | --- |

SECTION 7 – AS-BUILT DOCUMENTATION

| Field | Value |
| --- | --- |
| Firmware — Controller A (Before) |  |
| Firmware — Controller A (After) |  |
| Firmware — Controller B (Before) |  |
| Firmware — Controller B (After) |  |
| Actual NDU Start Time |  |
| Actual NDU End Time |  |
| Total I/O Interruption (min) |  |

## Post-Patch Command Output

Paste output of: SAN firmware version screenshot  |  esxcli storage nmp device list  |  backup console screenshot

SECTION 8 – SIGN-OFF

| Role | Name | Date / Time | Initials |
| --- | --- | --- | --- |
| Executing Engineer |  |  |  |
| Patching Manager |  |  |  |
| Customer POC | John Doe |  |  |

SECTION 9 – ISSUES AND LESSONS LEARNED