Quick Reference Card — VMware ESXi

Essential Patching Commands — Field Engineer Reference

Technician

| Document ID | MSP-TEC-001 |
| --- | --- |
| Category | Technician |
| Version | 1.0 |
| Audience | Field / Infrastructure Engineer |

# Command Reference  —  Test Customer Inc.

Commands most commonly needed during patching maintenance windows. Keep this open alongside the runbook during the maintenance window.

| Action | Command |
| --- | --- |
| Check ESXi version | vmware -v |
| List installed VIBs | esxcli software vib list |
| Install offline bundle | esxcli software vib install -d /vmfs/volumes/<ds>/<bundle>.zip |
| Update image profile | esxcli software profile update -d /vmfs/volumes/<ds>/<bundle>.zip -p <profile> |
| List image profiles in bundle | esxcli software sources profile list -d /vmfs/volumes/<ds>/<bundle>.zip |
| Reboot host | esxcli system shutdown reboot -r 'Patch' |
| List datastores | esxcli storage filesystem list |
| List running VMs | esxcli vm process list |
| Check HA config | vim-cmd hostsvc/hostsummary ¦ grep -i ha |
| Check multipath | esxcli storage nmp device list |
| Enable SSH (via vSphere API) | vim-cmd hostsvc/enable_ssh |
| Show host hardware info | esxcli hardware platform get |

# Pre-Patch Checks — Quick List

- Approved CR on file — confirmed

- Customer POC notified and available

- Backups/snapshots current (< 24 hours)

- OOB / console access confirmed

- Patch bundle downloaded and checksum verified

- Monitoring alerts suppressed

- NOC notified — window open

# Post-Patch Checks — Quick List

- New version/build confirmed

- All services / interfaces up

- HA / cluster sync confirmed (if applicable)

- Validation tests passed

- Monitoring alerts re-enabled

- NOC notified — window closed

- Customer POC sign-off obtained

- Runbook completed and filed

# Escalation Quick Reference

| Situation | Who to Call | When |
| --- | --- | --- |
| Rollback not working within 30 min | Patching Manager | Immediately |
| Device fails to boot after patch | Patching Manager + Vendor TAC | Within 15 min of confirmed boot failure |
| Data appears inaccessible (SAN/SQL) | Patching Manager — P1 Incident | Immediately |
| Customer escalation during window | Account Manager | Immediately |
| Maintenance window overrunning by > 50% | Patching Manager | Before exceeding window end time |