Work Instructions

Network Switches

Patching Procedure — MSP Managed Infrastructure

| Document ID | MSP-WI-SW |
| --- | --- |
| Device Type | Network Switches |
| Audience | Mid-Level Engineer |
| SOP Reference | MSP-PAT-002 |
| Runbook Reference | MSP-RB-SW |
| Version | 1.0 |

# Overview  —  Test Customer Inc.

These work instructions cover patching Cisco IOS/IOS-XE network switches. The process differs based on whether the switch runs in Bundle Mode (traditional, single .bin file) or Install Mode (modular, used on IOS-XE 16.x+). You must determine the mode before starting — the commands are different, and using the wrong approach can cause issues.

## Key Concepts

- Bundle Mode vs Install Mode:  Bundle Mode boots directly from a .bin image file. Install Mode uses a package directory structure and supports in-service upgrades on some platforms. IOS-XE 16.x+ defaults to Install Mode on newer hardware. Run 'show version' and check the boot variable — if it references a .bin file, you're in Bundle Mode. If it references 'packages.conf', you're in Install Mode.

- Spanning Tree:  Spanning Tree Protocol (STP) prevents loops by blocking redundant paths. When a switch reboots, it goes through STP convergence (30–50 seconds with classic STP, seconds with RSTP). During this time, connected devices may lose connectivity. Know your STP topology before patching.

- Console Access is Essential:  If the IOS upgrade fails and the switch does not boot, you need console access to reach ROMMON and recover. Never patch a switch you cannot reach via console or OOB.

- Stack vs Standalone:  Stacked switches (Cisco StackWise) can be upgraded simultaneously — the stack coordinates the upgrade internally. Standalone switches upgrade one at a time.

- VSS / StackWise Virtual:  Virtual Switch System and StackWise Virtual allow two physical chassis to act as one logical switch. These have specific upgrade procedures — consult vendor documentation.

## Tools and Access Required

- Console or OOB access to the switch

- SSH client for remote access

- TFTP server (or SCP/SFTP server) reachable from the switch management interface

- IOS / IOS-XE image file downloaded from Cisco Software Download Center

- MD5 checksum from Cisco for the downloaded image

- Network topology diagram showing uplinks and STP root placement

# 1. Determine Your Upgrade Mode and Obtain Image

Before downloading anything, determine whether your switch runs in Bundle Mode or Install Mode. This dictates which commands you will use.

- SSH into the switch and run:

show version | include Mode

— or —

show version | include INSTALL

If output shows 'INSTALL mode' or boot variable points to 'packages.conf' → Install Mode.

If boot variable points to a .bin file → Bundle Mode.

- Also run:

show boot

Note the current boot image path.

- Log in to Cisco Software Download Center (software.cisco.com). Navigate to your switch model and IOS/IOS-XE version.

- Download the correct image:

• Bundle Mode: download the .bin file

• Install Mode: download the .bin file (the install process converts it to packages)

- Verify the MD5 checksum using PowerShell:

Get-FileHash <image>.bin -Algorithm MD5

Compare to Cisco's listed MD5 value — they must match exactly.

WARNING: Using a corrupted or wrong IOS image is one of the most common causes of switches entering ROMMON after upgrade. Always verify the checksum before copying to the switch.

# 2. Back Up Configuration and Stage Image

Back up the running configuration and copy the firmware image to the switch before the maintenance window opens.

- Save the running configuration to your TFTP server:

copy running-config tftp://<server-ip>/<hostname>-pre-patch.cfg

Confirm the transfer completes with no errors.

- Copy the new IOS image to the switch flash:

copy tftp://<server-ip>/<image>.bin flash:

— or via SCP:

copy scp://<user>@<server-ip>/<path>/<image>.bin flash:

This may take several minutes depending on file size.

- After the copy completes, verify the image on the switch:

verify /md5 flash:<image>.bin

Compare the output MD5 to the expected value. This step confirms the image was not corrupted during transfer.

TIP: Stage the image to the switch during business hours if possible — before the maintenance window. The copy itself does not cause any disruption and can take 5–15 minutes on slower management interfaces.

# 3A. Upgrade — Bundle Mode

Use these steps if the switch runs in Bundle Mode (boot variable points to a .bin file).

- Set the new image as the boot image:

conf t

boot system flash:<new-image>.bin

no boot system flash:<old-image>.bin

end

- Verify the boot variable is correct:

show boot

Confirm the new image is listed as the primary boot image.

- Save the configuration:

write memory (or copy running-config startup-config)

- Schedule or initiate the reload:

• Scheduled: reload at HH:MM (24hr format, must be within maintenance window)

• Immediate: reload

Confirm with 'y' when prompted.

- Monitor the console during the reload. Watch for boot messages — the switch should display the new IOS version during boot.

- After reload, verify:

show version | include Version

# 3B. Upgrade — Install Mode (IOS-XE 16.x+)

Use these steps if the switch runs in Install Mode (boot variable points to packages.conf).

- Add the new image to the install repository:

install add file flash:<new-image>.bin activate commit prompt-level none

OR as separate steps:

install add file flash:<new-image>.bin

(Wait for completion, then:)

install activate

(Switch will reload automatically)

- The 'install activate' command will cause the switch to reload automatically. Monitor the console.

- After reload, verify the new version:

show version | include Version

- Commit the install (makes it permanent — rollback is no longer possible after commit):

install commit

WARNING: Do not run 'install commit' until you have fully validated the upgrade. Before committing, you can roll back with: install rollback to committed. After commit, rollback to the previous version requires manually loading the old image.

- Clean up old packages to free flash space:

install remove inactive

# 4. Post-Upgrade Validation

- Confirm new IOS version:

show version | include Version

- Verify spanning tree topology is stable and unchanged:

show spanning-tree

— Compare root bridge and port states to your pre-patch baseline documentation.

- Verify all interfaces are in their expected state:

show interfaces status

— All previously connected ports should show 'connected'. No previously active port should be 'notconnect'.

- Verify VLANs are intact:

show vlan brief

- Verify routing table if this is a Layer 3 switch:

show ip route

- Run a ping test between devices on different VLANs through the switch to confirm end-to-end connectivity.

- For stacked switches: verify all stack members are present:

show switch

— All members should show 'Ready' state.

# 5. ROMMON Recovery (Worst Case)

If the switch fails to boot and drops into ROMMON mode, use these steps to recover. This requires console access.

- At the ROMMON prompt, verify the flash contents:

dir flash:

- If the old image is still present, boot from it:

ROMON> boot flash:<old-image>.bin

- If flash is empty or corrupt, use TFTP boot:

ROMON> IP_ADDRESS=<switch-ip>

ROMON> IP_SUBNET_MASK=<mask>

ROMON> DEFAULT_GATEWAY=<gateway>

ROMON> TFTP_SERVER=<server-ip>

ROMON> TFTP_FILE=<image>.bin

ROMON> tftpdnld

ROMON> boot

WARNING: ROMMON recovery via TFTP requires a direct connection — the switch is not fully operational at this point. Use a laptop connected directly to the switch's management port or the console.

# 6. Common Issues and Troubleshooting

| Symptom | Likely Cause | Resolution |
| --- | --- | --- |
| Switch drops to ROMMON after reload | Boot variable points to non-existent image | Boot from old image via ROMMON. Recopy new image. Reset boot variable correctly. |
| STP topology changed after patch | Port came up in different STP role | Run 'show spanning-tree' and compare to baseline. If root bridge changed, check port priority settings. PortFast/BPDU Guard settings should be preserved across upgrades. |
| Interface shows notconnect after upgrade | Port was disabled in config or hardware issue | Check 'show interfaces <int> status'. Run 'no shutdown' if administratively down. Reseat cable if physical issue. |
| 'no space' error when copying image | Flash is full | Delete old images: delete /force flash:<old-image>.bin. Or use install remove inactive (Install Mode). Verify with 'dir flash:' before retrying. |
| Install activate hangs | Image incompatibility or hardware fault | Wait up to 30 min before assuming stuck. Check console for error messages. Contact Cisco TAC if stuck longer than 45 min. |